报告：基于改进LSTM模型的数据篡改攻击识别研究

日期：20260514

1　研究背景

新型电力系统运维网关作为电力信息物理系统的关键交互节点与安全屏障，其自身安全对保障电网稳定运行至关重要。近年来，针对此类网关的数据篡改攻击手段日趋复杂与隐蔽。攻击者往往通过高级持续性威胁等方式，在长时间跨度内隐匿攻击行为，并可能伴随数据包丢弃或篡改，导致监测时序数据出现缺失与异常。使得传统基于规则或静态模型的检测方法，在面对动态隐匿的攻击路径、多尺度退化的时序特征以及极端稀疏的异常样本时，检测灵敏度不足、误报率高且难以适应实际应用中嵌入式环境的实时性要求[1]。因此，针对电力系统运维网关的攻击问题，设计一种能够高效识别数据篡改攻击的自适应检测机制，已成为一个亟待解决的学术与工程难题。

现有研究：一是基于传统机器学习与特征工程的方法，如文献[2]基于最大相关-最小冗余特征选择方法提取信息攻击事件最优表征特征子集，利用极限梯度提升树进行智能电网信息攻击的识别，文献[3]采用卡尔曼滤波方法用于智能电网数据注入攻击等的检测和识别，但这类方法对时序动态特征的深层依赖关系建模能力有限，难以捕捉攻击的隐匿性演化过程；二是基于信号处理与信息论的方法，如文献[4]通过基于滑动窗口的多尺度熵对序列频谱进行分析，进而识别数据篡改攻击，但其固定的分析窗口难以适配流式数据并有效区分类似噪声的微弱攻击跃变；三是基于深度学习融合模型的方法，如文献[5]通过组合卷积神经网络、双向门控循环单元和极限梯度提升树等方法用以识别虚假数据注入攻击篡改电网量测信息，但这类模型复杂度高，在实际应用中类似MPC5748G的嵌入式硬件上部署面临实时推断与资源占用的瓶颈。此外，文献[6]将图神经网络应用于智能电网虚假数据注入攻击，该类方法虽能建模系统拓扑关系，但对纯时序数据中的点异常与连续缺失场景响应不足，且在类别极端不平衡时易产生模型偏置；文献[7]和文献[8]针对智能电网中的虚假数据注入攻击，均利用深度神经网络中的LSTM模型进行数据完整性和恶意攻击检测，这些模型能够有效识别潜在的攻击场景，但针对动态攻击的适应性较弱。综上所述，现有方法在应对新型电力系统运维网关特有的嵌入式环境约束、IEC 61850协议栈下多源异构时序流以及伴随数据缺失的复合型篡改攻击这一综合挑战时，仍存在明显局限：或缺乏对长程时序依赖与微弱异常的有效捕捉，或模型复杂难以落地，或对样本不平衡及数据缺失鲁棒性不足。

针对上述问题，本文提出一种面向新型电力系统运维网关基于改进LSTM模型的时序数据篡改攻击识别方法，所述方法针对实际系统中MPC5748G与IEC 61850架构下的网关数据篡改攻击识别问题，通过结合在线学习与加权损失机制提升对网关不平衡数据的适应性，并利用动态滑动窗口增强对网关攻击序列的辨识能力。所述方法主要对传统的LSTM模型进行如下改进：针对实际系统中嵌入平台的应用，采用样本增量学习策略对模型参数进行实时更新；在模型损失函数中加入电力系统物理规律约束加权惩罚项；采用动态滑动窗口对攻击数据序列进行识别。实验结果表明，所述改进模型能够有效提高对于电力系统运维网关数据篡改攻击的识别效果，进而为电力系统的运维提供支持和保障。

2　研究方法：基于改进LSTM的运维网关动态攻击辨识方法

LSTM网络通过遗忘门、输入门、输出门等门控机制和细胞状态，能够有效捕捉电力时序数据中的长期依赖关系以及数据篡改攻击导致的数据分布异常跃变[9-10]。其核心在于对信息流的自适应调控：遗忘门选择性过滤历史冗余信息，输入门与候选记忆单元协同捕捉当前输入的异常特征，输出门则控制哪些关键状态信息被传递用于攻击判定。三者通过细胞状态的时序更新，实现了对攻击状态的动态跟踪。

LSTM模型工作流程：

步骤1：前馈输入。在每一时间步t，模型从运维网关实时采集多维时序数据向量xtRd作为输入。其中，d为特征维度，通常包括A相电压Ut、电流It、有功功率Pt等电气量，以及设备状态码等状态量。该向量承载了网关在t时刻的完整运行状态信息。

步骤2：门控信号计算与记忆更新。神经元的输出用于表征当前时间步网关数据的状态特征yt，其计算公式如下：

（1）

式（1）中，zt为输出门，用于控制记忆单元中哪些关键特征需要传递到当前输出以区分网关正常数据与篡改攻击数据；tanh(xt)将记忆单元的数值压缩至[-1,1]区间，避免因网关数据数值范围差异过大导致模型训练不稳定[11]。

遗忘门负责过滤冗余信息，聚焦攻击特征。在网关时序数据中，存在大量冗余信息，遗忘门的作用是选择性遗忘记忆单元中无关的冗余信息，保留与攻击识别相关的数据突变特征，其计算公式为：

（2）

式（2）中，()为sigmoid激活函数，输出值介于[0,1]之间，值越接近1表示对应特征对攻击识别的重要性越高[12]；at为当前时间步网关输入数据； yt-1为上一时间步的隐含层输出，携带了t-1时刻网关数据的状态信息[13]；为输入权重，用于调节当前输入数据各维度特征贡献度[14]；为循环权重，控制上一时刻状态信息对当前输出的影响，适配网关数据时序连续性[15]；E为对角矩阵，用于避免记忆单元中传感器微小噪声对输出产生干扰，提升模型对攻击特征的敏感度[16]。

输入门与候选记忆单元负责捕捉攻击导致的时序异常，输入门用于控制当前时间步的新特征（电压异常跳变）是否纳入记忆单元，候选记忆单元则用于生成当前时间步的新特征表示，二者共同实现对网关数据篡改攻击时序异常的捕捉。候选记忆单元，计算公式为：

（3）

式（3）中，in和out分别为候选记忆单元的输入权重与循环权重，用于将当前输入数据与上一时刻状态融合，生成新的特征表示[17-18]。

步骤3：记忆单元更新。记忆单元的更新是LSTM适配网关时序数据的核心环节，通过遗忘门过滤冗余信息、输入门纳入新特征，实现对网关数据从正常到攻击状态变化的动态跟踪，公式如下：

（4）

在网关正常运行时，遗忘门ft接近1、输入门接近0，记忆单元主要保留历史正常数据特征[19-20]；当发生数据篡改攻击时，输入门迅速升高至接近1，将攻击导致的异常特征纳入记忆单元，同时遗忘门ft适当降低，保留部分历史正常特征以对比区分，实现对攻击的快速响应[21-22]。

步骤4：输出生成。输出门的核心作用是从记忆单元中筛选出对攻击识别有价值特征。输出门负责从记忆单元中筛选对攻击识别有关键区分价值的特征，如篡改攻击导致的功率数据异常偏差[23-24]、正常数据的平稳变化特征[25]等。

步骤5：攻击状态判定。最终输出结果用于表征当前时间步网关数据的状态特征，为后续攻击判定提供依据。

基于LSTM的数据纂改攻击识别模型常实时运行于网关节点的嵌入式平台，传统批量离线训练模式难以适应[26-27]运维网关数据实时流式到达与攻击样本极端稀疏的双重挑战。因此，针对电力系统实践中嵌入式平台的应用，文中在前述LSTM模型的基础上进行了如下进一步改进：增加了嵌入式平台在线单样本增量学习机制；在模型损失函数中融合电力系统物理规律约束；使用滑动窗口进行数据纂改攻击序列识别，从而增加模型对电力系统动态数据环境的自适应能力，并提升对稀少攻击样本的识别效果。

（1）在线增量学习

对LSTM模型的第一个改进是利用在线增量学习机制实时更新模型参数，其核心思想是：网关嵌入式平台中所部署的模型能够利用每一个新到达的样本(xt, yt)即时更新其参数，而无需存储历史数据或进行批量再训练。该机制由以下三个关键部分组成：

（1）样本到达触发更新：一旦网关采集并预处理完一个时刻t的数据样本，该样本即作为增量学习的输入，触发一次模型参数更新。保证了模型对数据分布变化的响应延迟最低。

（2）基于时序平滑的梯度更新策略：为防止单个样本引起参数剧烈震荡，采用结合了动量的随机梯度下降进行更新。参数更新公式不仅考虑当前样本的梯度，也纳入历史更新方向，以保持学习的稳定性：

（5）

式（5）中，t为t时刻的模型参数，为损失函数，为学习率，为动量系数，Vt为累积更新动量。当连续检测到攻击样本时，临时提高，加速模型学习攻击特征；当长期为正常样本时，逐步衰减，避免对稳定模式的过拟合。

（3）模型参数轻量化与定点化：为适配MPC5748G等嵌入式平台的算力与存储限制，对LSTM模型进行剪枝与量化，将权重与激活值转换为定点数。增量学习过程中的梯度计算与参数更新均在定点数域进行，确保机制在实际硬件上的运行效率。

（2）融合物理规律约束的加权损失函数

对LSTM模型的第二个改进是在损失函数中加入融合物理规律约束的加权惩罚项，电力系统在运行过程中内部各设备的物理参数受各自物理规律支配，正常工况下的运行数据分布遵循这些规律，如电压限制、电流限制、功率限制等，当发生数据注入及篡改攻击时，通常所加入的篡改数据会违背系统的物理规律，产生矛盾。为此，在LSTM模型的交叉熵损失函数中进一步加入违背电力系统运行物理规律的惩罚项，使LSTM模型在训练过程中同时学习数据样本的特点及各样本所需遵循的系统物理约束。定义对于样本(xt, yt)，损失函数的额外惩罚项定义为：

（6）

式（6）中，xlow和xup分别为数据样本的下限值和上限值（如系统中电压、电流等上下限）。当发生数据注入纂改攻击，纂改数据违背系统运行的物理规律约束时，即注入数据越限大于xup或小于xlow，该数据样本将产生较大的惩罚项；当网关中数据样本为正常时，损失函数则保持不变。

上述融合物理规律约束的加权惩罚项损失函数能提升LSTM模型在训练过程中对稀疏攻击样本识别能力，有效缓解样本不平衡带来的模型偏置问题。同时结合物理信息加权的损失函数设计与在线学习机制，能够有效适应网关数据的动态变化特性，并在样本极端不平衡条件下保持对篡改攻击的高灵敏度识别，为嵌入式运维网关提供了一种实时、自适应的安全防护解决方案。

3　简单实验

实验采用基于NXP MPC5748G微控制器的嵌入式网关原型系统作为算法部署与轻量级验证平台。该平台复现了真实网关的核心硬件架构：多核、通信接口、数据采集模块与IEC 61850协议栈，用于为攻击注入提供可控接口其并验证所提算法在资源受限的嵌入式环境中的可部署性与实时性。该实验平台的主要配置如下：主控单元为NXP MPC5748G (双核锁步架构，主频200MHz，具备ASIL-D功能安全等级)，攻击注入通过以太网接口模拟外部攻击渗透，状态量数据中设备状态码以“00”表示正常，时间戳遵循IEEE 1588精密时钟协议(PTP)同步，确保时序一致性。所模拟的网关数据被篡改情况如表1所示。

表1 模拟网关数据篡改攻击

Tab.1 Simulated gateway data tampering attack

| 攻击环节 | 具体操作描述 |
| --- | --- |
| 入侵与绕过校验 | 攻击者非法侵入网关数据传输与存储模块，绕过数据校验机制 |
| 篡改时间选择 | 结合电力系统每10min一次的典型数据采集周期，选择在时间14:30这一监测节点进行篡改。此时处于用电高峰过渡期，主站系统数据处理负载较高，利于虚假数据隐藏 |
| 篡改实施过程 | 网关正准备将110kV线路A相电压正常监测值打包上传至主站系统，攻击者通过预先植入的恶意程序拦截数据帧，在数据加密传输前的窗口期，对电压字段的二进制存储值进行位运算修改，篡改操作持续约2s |
| 篡改 结果 | 虚假数据被封装并正常上传，将原本正常采集的110kV线路A相电压强制篡改为210kV，制造电压偏低的虚假状态 |

文中所述基于改进LSTM模型的电力系统运维网关数据篡改攻击识别的训练过程损失函数随训练次数的变化如图3所示。由图3可知，所述模型的训练损失函数随着训练次数的增加而减小，当训练次数接近3000时，模型训练过程的损失函数逐渐趋于稳定，表明此时所述模型的网络参数逐渐趋于收敛，对输入训练数据分布和特征的学习逐渐完成。

图3 模型训练过程损失变化

Fig.3 Loss during model training

在上述训练所得到模型的基础上，针对表1中的网关数据篡改攻击情形，进一步对比分析了文中基于改进LSTM模型的攻击识别方法（文中方法）和基于极限梯度提升树算法（方法1）、基于多尺度熵识别方法（方法2）、基于卷积神经网络-双向门控循环单元-极限梯度提升树识别方法（方法3）、基于数据驱动识别方法（方法4）的识别效果。对比过程所使用的评估指标包括精确率（Precision）、召回率（Recall）和F1-score，结果如表2所示。由表2可知所述方法的精确率、召回率和F1-score指标分别为0.974、0.966和0.973，对数据纂改攻击识别的三类指标均优于对比方法，表明文中所述方法对于数据纂改攻击具有较好的识别性能。

表2 不同方法的识别性能

Tab. 2 Performance of different methods

|  | 方法1 | 方法2 | 方法3 | 方法4 | 文中方法 |
| --- | --- | --- | --- | --- | --- |
| 精确率 | 0.913 | 0.930 | 0.945 | 0.911 | 0.974 |
| 召回率 | 0.920 | 0.922 | 0.937 | 0.920 | 0.966 |
| F1-score | 0.914 | 0.931 | 0.942 | 0.917 | 0.973 |

参 考 文 献

[1] 张程彬,崔明建,张梓枭,等.考虑攻击偏好的三相不平衡配电系统分布式FDIA检测[J].电力系统保护与控制, 2024, 52(24):109-119.

Zhang Chengbin, Cui Mingjian, ZHANG Zixiao,et al. Distributed FDIA detection for three-phase unbalanced distribution systems considering attack preferences [J]. Power System Protection and Control, 2024, 52(24):109-119.

[2] 邬蓉蓉,黎新,宾冬梅.基于XGBoost算法的智能电网信息攻击识别模型[J].电测与仪表, 2023, 60(1):64-70.

Wu Rongrong, Li Xin, Bin Dongmei. Network attack identification model of smart grid based on XGBoost algorithm [J]. Electrical Measurement & Instrumentation, 2023, 60(1):64-70.

[3] K. MANANDHAR, X.J.CAO, F.HU. et al. Detection of faults and attacks including false data injection attack in smart grid using kalman filter[J]. IEEE Transactions on Control of Network System, 2014, 1(4)：370-379.

[4] 王安义,龚健超,朱涛.基于多尺度熵的抗频谱感知数据篡改攻击协作频谱感知方法研究[J].电子与信息学报, 2025, 47(5):1-9.

Wang Anyi, Gong Jianchao, Zhu Tao. Cooperative Spectrum Sensing Method Against Spectrum Sensing Data Falsification Attacks Based on Multiscale Entropy [J]. Journal of Electronics & Information Technology, 2025, 47(5):1-9.

[5] 黄冬梅,杨旭,胡安铎,等.基于CNN-BiGRU-XGBoost的新型电力系统虚假数据注入攻击检测[J].电网技术, 2025, 49(5):2119-2127.

Huang Dongmei, Yang Xu, Hu Anduo,et al. Detection of False Data Injection Attack in New Power System Based on CNN-BiGRU-XGBoost [J]. Power System Technology, 2025, 49(5):2119-2127.

[6] 陈柏任,夏候凯顺,李梦诗.基于数据驱动的电力系统虚假数据注入攻击防御框架的研究[J].电测与仪表, 2024, 61(12):10-16.

Chen Bairen, Xiahou Kaishun, Li Mengshi. Research on defense framework for false data injection attacks in power system based on data-driven algorithm [J]. Electrical Measurement & Instrumentation, 2024, 61(12):10-16.

[7] Debottam Mukherjee, Samrat Chakraborty, Almoataz Y. Abdelaziz, et al. Deep learning-based identification of false data injection attacks on modern smart grids [J]. Energy Reports, 2022, 8(15): 919-930.

[8] Y. He, G. J. Mendis and J. Wei. Real-Time Detection of False Data Injection Attacks in Smart Grid: A Deep Learning-Based Intelligent Mechanism [J]. IEEE Transactions on Smart Grid, 2017, 8(5): 2505-2516.

[9] 许克珂.基于公平秘密共享的多节点安全设备日志防篡改协议研究[J].电子技术应用, 2024, 50(1):179-185.

Xu Keke. Research on multi-node security device log tamper-proof protocol based on fair secret sharing [J]. Application of Electronic Technique, 2024, 50(1):179-185.

[10] 李明月,毕秀丽,刘波.新颖的内容自适应图像篡改检测与自恢复算法研究[J].重庆邮电大学学报（自然科学版）, 2024, 36(4):797-806.

Li Mingyue, Bi Xiuli, Liu Bo. Novel content-adaptive tampering detection and self-recovery algorithm study [J]. Journal of Chongqing University of Posts and Telecommunications(Natural Science Edition), 2024, 36(4):797-806.

[11] 王文博,刘绚,张博,等.基于协议特征的电力工控网络流量异常行为检测方法[J].电力系统自动化, 2023, 47(02):137-145.

Wang Wenbo, Liu Xuan, Zhang Bo,et al. Protocol Characteristics Based Detection Method for Abnormal Flow Behavior in Electric Power Industrial Control Network [J]. Automation of Electric Power System, 2023, 47(02):137-145.

[12] 陈瑞政,李宁,郭艳,等.基于模糊提取器的无人机轻量级安全认证方法[J].计算机工程与设计, 2025, 46(5):1356-1363.

Chen Ruizheng, Li Ning, Guo Yan, et al. Lightweight safety authentication method of UAV based on fuzzy extractor [J]. Computer Engineering and Design, 2025, 46(5):1356-1363.

[13] 冯睿琪,王雷蕾,林翔,等.基于软件防护扩展的车联网路况监测安全数据处理框架[J].计算机应用, 2023, 43(6):1870-1877.

Feng Ruiqi, Wang Leilei, Lin Xiang, et al. Software Guard Extensions-based secure data processing framework for traffic monitoring of internet of vehicles [J]. journal of Computer Applications, 2023, 43(6):1870-1877.

[14] 许爽,刘智颖,李元诚,等.针对电池储能系统假数据注入攻击的智能化检测方法研究[J].中国电机工程学报, 2023, 43(17):6628-6638.

Xu Shuang, Liu Zhiying, Li Yuancheng,, et al. Research on Intelligent Detection Methods for False Data Injection Attacks on Battery Energy Storage Systems [J]. Proceedings of the CSEE, 2023, 43(17):6628-6638.

[15] 谢宏,张华赢,梁晓锐,等.基于关系图卷积神经网络的新能源配电台区拓扑识别方法[J].电测与仪表, 2024, 61(7):94-102.

Xie Hong, Zhang Huaying, Liang Xiaorui, et al. A topology identification method based on relational-graph convolutional network for distribution substation area with high renewables [J]. Electrical Measurement & Instrumentation, 2024, 61(7):94-102.

[16] 苑申昊,陈增茂,刁鸣,等.基于复合熵和证据理论的抗SSDF攻击协作频谱感知方法[J].系统工程与电子技术, 2023, 45(11):3663-3670.

Yuan Shenhao, Chen Zengmao, Diao Ming,et al. Cooperative spectrum sensing method against SSDF attacks based on composite entropy and evidence theory [J]. Systems Engineering and Electronics, 2023, 45(11):3663-3670.

[17] 于枫,孟令辉,彭家辉,等.一种基于区块链的物联网数据安全交易方案[J].广西师范大学学报:自然科学版, 2023, 41(4):84-95.

Yu Feng, Meng Linghui, Peng Jiahui,et al. A Blockchain-Based IoT Data Security Transaction Scheme [J]. Journal of Guangxi Normal University:Natural Science Edition, 2023, 41(4):84-95.

[18] 陈昌盛,陈自炜,李锡劲.基于文字边缘失真特征的翻拍图像篡改定位[J].中国科技论文, 2024, 19(2):160-168.

Chen Changsheng, Chen Ziwei, Li Xijin Tampering localization of recaptured image based on text edge distortion features [J]. China Sciencepaper, 2024, 19(2):160-168.

[19] 田秀霞,刘正,刘秋旭,等.一种改进Faster R-CNN的图像篡改检测模型[J].计算机工程与科学, 2023, 45(6):1030-1039.

Tian Xiuxia, Liu Zheng, Liu Qiuxu,et al. An image tampering detection model based on improved Faster R-CNN [J]. Computer Engineering & Science, 2023, 45(6):1030-1039.

[20] 张旭,王旭东,何鑫,等.基于改进孤立森林的电力系统碳交易数据异常检测[J].电测与仪表, 2025, 62(6):218-224.

Zhang Xu, Wang Xudong, He Xin, et al. Anomaly detection for carbon trading data in power system based on improved isolation forest method [J]. Electrical Measurement & Instrumentation, 2025, 62(6):218-224.

[21] 付伟,谢振杰.基于国密算法的轻量级跳变密钥加密系统[J].海军工程大学学报, 2023, 35(2):18-24.

FU wei, XIE Zhenjie. A lightweight hopping key encryption system based on domestic cryptographic algorithm [J]. Journal of Naval University of Engineering, 2023, 35(2):18-24.

[22] 倪雪莉,马卓,王群.区块链P2P网络及安全研究[J].计算机工程与应用, 2024, 60(5):17-29.

Ni Xueli, Ma Zhuo, Wang Qun. Research on Blockchain P2P Network and Its Security [J]. Computer Engineering and Applications, 2024, 60(5):17-29.

[23] 陈媛媛,赵秉琨,王慧,等.基于LSTM模型的时序InSAR地表形变预测[J].人民长江, 2024, 55(3):146-152.

Chen Yuanyuan, Zhao Bingkun, Wang Hui,et al. Time-series InSAR ground deformation prediction based on LSTM model [J]. Yangtze River 2024, 55(3):146-152.

[24] 齐立忠,张苏,武宏波,等.基于新型电力系统信息模型的重点区域高压输电线路损毁态势感知预警方法研究[J].电测与仪表, 2024, 61(6):133-140.
Qi Lizhong, Zhang Su, Wu Hongbo, et al. Research on awareness warning method for damage situation of key regional high voltage transmission lines based on novel power system information model[J]. Electrical Measurement & Instrumentation, 2024, 61(6):133-140.

[25] 黄启萌,吴苗苗,李云.对抗逃避攻击的过滤式对抗特征选择研究[J].电信科学, 2023, 39(7):46-58.

Huang Qimeng, Wu Miaomiao, Li Yun. Research on filter-based adversarial feature selection against evasion attacks [J]. Telecommunications Science, 2023, 39(7):46-58.

[26] 张静克,杨凯,李超,等.基于先验知识嵌入LSTM-PPO模型的智能干扰决策算法[J].通信学报, 2024, 45(12):227-239.

Zhang Jingke, Yang Kai, Li Chao,et al. lligent interference decision algorithm with prior knowledge embedded LSTM-PPO model [J]. Journal on Communications, 2024, 45(12):227-239.

[27] 张钦,郑尚,邹海涛,等.一种融合注意力机制的形变LSTM智能代码补全方法[J].小型微型计算机系统, 2024, 45(2):498-504.

Zhang Qin, Zheng Shang, Zou Haitao,et al. Intelligent Code Completion Based on Mogrifier LSTM and Attention Mechanism [J]. Journal of Chinese Computer Systems, 2024, 45(2):498-504.